UR3e Dual Robot System

Setup & Startup Guide

Hande Gripper  •  Intel RealSense Depth Cameras  •  ROS Melodic  •  Webots 2021a

# System Overview

This guide covers the complete setup and operation of a dual UR3e robot system with Hande grippers and Intel RealSense depth cameras. The system uses a hybrid architecture:

| Component | Description |
| --- | --- |
| Ubuntu VM (18.04) | Runs ROS, Webots simulation, and robot drivers. Interfaces directly with robot hardware via ethernet. |
| Windows Host | Runs the GPU inference server (gpu_server.py) which handles neural network computation. |
| Robot 1 | UR3e at 192.168.1.120 — connects via dedicated Ethernet adapter. |
| Robot 2 | UR3e at 192.168.1.112 — connects via same Ethernet network. |
| Cameras | Intel RealSense D455 & D415 cameras, plugged into the host laptop/Raspberry Pi via USB. |

# Part 1: One-Time Setup

Complete these steps once. You do not need to repeat them for each session.

## 1.1  Install the Ubuntu VM

- Download and install VMware Workstation on Windows.

- Create a new VM and install Ubuntu 18.04 onto it. (Recommend VMware Workstation)

- Clone the project repository inside the VM:

| $ | git clone https://github.com/<your-repo>/UR3e_Hybrid_System.git |
| --- | --- |

- Download Webots version 2021a from the official releases page:

https://github.com/cyberbotics/webots/releases

- Run the setup script to install all ROS packages and dependencies:

| $ | cd <path>/UR3e_Hybrid_System/vm_simulation_system chmod +x setup.sh ./setup.sh |
| --- | --- |

## 1.2  Configure VM Networking

The VM needs two network adapters: one for internet access (Wi-Fi/GPU connection) and one dedicated to the robot Ethernet network. Follow these steps carefully.

### Step A — Bridge Ethernet to VMnet0

- In Windows, open the Start menu and search for Virtual Network Editor (installed with VMware Workstation).

- Click Change Settings in the bottom-right corner (requires admin privileges).

- Select VMnet0 at the top. It likely shows "Bridged (Auto)".

- In the "Bridged to:" dropdown, change it from Automatic to your physical Ethernet adapter (usually named Realtek, Intel, or Killer PCIe GbE).

- Click Apply, then OK.

### Step B — Add a Second Network Adapter to the VM

⚠ NOTE: Power off your VM before making these changes.

- In VMware Workstation, right-click your Ubuntu VM and select Settings.

- Your existing Network Adapter 1 handles internet — leave it untouched.

- Click Add... at the bottom, select Network Adapter, and click Finish.

- Click on the new Network Adapter 2, select Custom: Specific virtual network, and choose VMnet0 (Bridged) from the dropdown.

- Click OK.

### Step C — Create a Dedicated Robot Bridge (VMnet2)

- Reopen Virtual Network Editor and click Add Network....

- Select VMnet2 from the list and click OK.

- Click VMnet2 to highlight it, then select the Bridged radio button.

- In the "Bridged to:" dropdown, select your physical Ethernet adapter (same adapter used in Step A).

- Click Apply, then OK.

- In VM Settings, add another Network Adapter following the same process as Step B, but select VMnet2 instead.

## 1.3  Windows Firewall — Open the GPU Server Port

The GPU server communicates over port 8888. Add a Windows Firewall inbound rule to allow this traffic.

| $ | netsh advfirewall firewall add rule name="UR3e GPU Server" dir=in action=allow protocol=TCP localport=8888 |
| --- | --- |

Verify connectivity between the VM and host:

| $ | ping <HOST_IP> # From VM to Windows host telnet <HOST_IP> 8888 # Test GPU server port |
| --- | --- |

## 1.4  Install Windows Python Dependencies

| $ | cd {path}\host_gpu_system python3 -m venv venv venv\Scripts\activate pip install -r requirements.txt |
| --- | --- |

## 1.5  Install URCaps on the Robot Pendants

Each UR3e pendant requires two URCap plugins. Download and transfer them via USB:

- RS-485 Tool Comm: rs485.urcap

https://github.com/UniversalRobots/Universal_Robots_ToolComm_Forwarder_URCap/releases

- External Control: externalcontrol.urcap

https://github.com/UniversalRobots/Universal_Robots_ExternalControl_URCap/releases

💡 TIP: If the releases don’t show, try clicking on the page and using Crtl + Shift + R

### Configure External Control on Each Pendant

- Create a blank program on the pendant.

- Under URCaps, add External Control and ensure it is set to loop.

- Navigate to Installation → URCaps → External Control.

- Set the Host IP to your VM's IP address.

- Robot 1: set the custom port to 50002.

- Robot 2: set the custom port to 50012.

## 1.6  Robot Calibration (First-Time Only)

⚠ NOTE: Only required the very first time you connect to each robot. The generated .yaml files are reused in all future sessions.

| $ | # Robot 1 roslaunch ur_calibration calibration_correction.launch \ robot_ip:=192.168.1.120 \ target_filename:=/home/YOUR_USERNAME/calibration_robot.yaml # Robot 2 roslaunch ur_calibration calibration_correction.launch \ robot_ip:=192.168.1.112 \ target_filename:=/home/YOUR_USERNAME/calibration_robot2.yaml |
| --- | --- |

# Part 2: Simulation (Webots)

Use this workflow for training and testing in the Webots simulator without physical robots.

## 2.1  Training Mode

Open five separate terminals in the order listed below.

### Windows — Start the GPU Server First

| $ | cd {path}\host_gpu_system venv\Scripts\activate python src/gpu_server.py |
| --- | --- |

### VM — Terminal 1: ROS Core

| $ | roscore |
| --- | --- |

### VM — Terminal 2: Webots

| $ | webots ~/catkin_ws/src/vm_simulation_system/Webots/worlds/Environmentnewww.wbt |
| --- | --- |

### VM — Terminal 3: Simulation Client — Robot 1

| $ | cd ~/catkin_ws/src/vm_simulation_system export WEBOTS_ROBOT_NAME="ur3e_robot" # Run indefinitely: python3 src/simulation_client.py --mode training --robot-id 1 # OR run a fixed number of episodes: python3 src/simulation_client.py --mode training --robot-id 1 --episodes <N> |
| --- | --- |

### VM — Terminal 4: Simulation Client — Robot 2

| $ | cd ~/catkin_ws/src/vm_simulation_system export WEBOTS_ROBOT_NAME="ur3e_robot2" python3 src/simulation_client.py --mode training --robot-id 2 |
| --- | --- |

💡 TIP: When stopping the simulation, always click Reset Simulation in Webots before closing. This correctly stops simulation_client.py and resets camera positions. Saving or restarting without resetting can cause cameras to shift from their original positions.

## 2.2  Inference Mode (Running the Trained Model)

roscore must already be running. Start the GPU server on Windows first, then launch the clients.

### Windows — GPU Server

| $ | cd {path}\host_gpu_system venv\Scripts\activate python src/gpu_server.py |
| --- | --- |

### VM — Terminal 3: Inference Client — Robot 1

| $ | cd ~/catkin_ws/src/vm_simulation_system export WEBOTS_ROBOT_NAME="ur3e_robot" python3 src/simulation_client.py --mode inference --robot-id 1 |
| --- | --- |

### VM — Terminal 4: Inference Client — Robot 2

| $ | cd ~/catkin_ws/src/vm_simulation_system export WEBOTS_ROBOT_NAME="ur3e_robot2" python3 src/simulation_client.py --mode inference --robot-id 2 |
| --- | --- |

## 2.3  Optional Client Flags

| Flag | Description |
| --- | --- |
| --free | Allows you to manually put the object anywhere. |
| --cycle <N> | Cycles through each phase, set the number of episodes per training phase. |
| --phase <N> | Stays at a specific training phase. |

# Part 3: Real Robot Operation

This section covers launching the full system with physical UR3e robots and RealSense cameras.

⚠ NOTE: Cameras must be plugged in via USB. Robots must be connected via dedicated Ethernet — Wi-Fi is too unstable and the drivers will disconnect immediately.

## 3.1  Launch Sequence

Start each component in the order below. Robot 1 and Robot 2 terminals can be started roughly in parallel, but the GPU server and roscore must come first.

### Windows — GPU Server

| $ | cd {path}\host_gpu_system venv\Scripts\activate python src/gpu_server.py |
| --- | --- |

### VM — Terminal 1: ROS Core

| $ | roscore |
| --- | --- |

### VM — Terminal 2: Robot 1 Driver (192.168.1.120)

| $ | source ~/catkin_ws/install_isolated/setup.bash roslaunch ~/robot1_bringup.launch |
| --- | --- |

💡 TIP: Start the External Control program on Robot 1’s pendant after this terminal is ready. To update the robot IP, run: nano ~/robot1_bringup.launch

### VM — Terminal 3: Robot 1 Gripper

| $ | source ~/catkin_ws/install_isolated/setup.bash ROS_NAMESPACE=ur3e_robot1 rosrun robotiq_2f_gripper_control \ Robotiq2FGripperRtuNode.py /tmp/ttyUR __name:=gripper_node |
| --- | --- |

### VM — Terminal 4: Robot 1 Camera

| $ | roslaunch realsense2_camera rs_camera.launch \ align_depth:=true \ depth_width:=1280 \ depth_height:=720 \ depth_fps:=30 |
| --- | --- |

### VM — Terminal 5: Robot 2 Driver (192.168.1.112)

| $ | source ~/catkin_ws/install_isolated/setup.bash roslaunch ~/robot2_bringup.launch |
| --- | --- |

💡 TIP: Start the External Control program on Robot 2’s pendant after this terminal is ready. To update the robot IP, run: nano ~/robot2_bringup.launch

### VM — Terminal 6: Robot 2 Gripper

| $ | source ~/catkin_ws/install_isolated/setup.bash ROS_NAMESPACE=ur3e_robot2 rosrun robotiq_2f_gripper_control \ Robotiq2FGripperRtuNode.py /tmp/ttyUR2 __name:=gripper_node |
| --- | --- |

### VM — Terminal 7: Robot 2 Camera

| $ | roslaunch realsense2_camera rs_camera.launch \ align_depth:=true \ depth_width:=1280 \ depth_height:=720 \ depth_fps:=30 \ camera:=camera2 |
| --- | --- |

### VM — Terminal 8 & 9: Inference Clients (launch simultaneously)

| $ | # Terminal 8 — Robot 1 cd ~/catkin_ws/src/vm_simulation_system source ~/catkin_ws/install_isolated/setup.bash python3 src/simulation_client.py --ros-camera --robot-id 1 --mode inference --free # Terminal 9 — Robot 2 cd ~/catkin_ws/src/vm_simulation_system source ~/catkin_ws/install_isolated/setup.bash python3 src/simulation_client.py --ros-camera --robot-id 2 --mode inference --free |
| --- | --- |

UR3e Dual Robot System — Setup & Startup Guide